SS 350HD Bull-Dog Installation

The Model SS 350HD Bull-Dog Hitch Accessory is manufactured for use with all Bull-Dog Trailer Hitch Couplers attached to Receiver adapters, but not for use with the ATWOOD MODEL 64-4102 with the 2 5/16” Trailer Hitch Coupler.

The universal kit design has 2 Step washers included in the SS 350HD Bull-Dog kit. Bull-Dog Trailer Ball heights usually need to be selected with Trailer Hitch Coupler clearances in mind; and because of this, the Step washer installation location needs to be determined by you as appropriate for you’re your clearance requirements.

To install, determine the size of the trailer ball shank, either 1” or 1 ¼” diameter. Select the Step washer that matches the shank size you are using and place through the top of the Safety Sentry Base mount plate or into the bottom of the Safety Sentry Base mount plate based on your Trailer Hitch Coupler clearance requirements or as you prefer if clearance requirements are determined to be a non-issue. Install the Trailer Ball and apply the lock washer.